# CU

## Parameters

- PARTIAL_SIZE.

- DATA_WIDTH.

- DATA_SIZE = PARTIAL_SIZE * DATA_WIDTH.

- NUMBER_OF_PARTIALS_PER_OUTPUT = 2.

## Input Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| clk | 1 | The clock |
| rst_n | 1 | Asynch active low reset |
| partial_valid | 1 | When high, means the partial_data going to the input sampler is valid |
| segment_step | 1 | When high, means the partial_data going to the input sampler is valid and we are going to process the next segment |
| phase_change | 1 | When high, means the partial_data going to the input sampler is valid and if we are in the INITIAL_PARTIALS state we change the state to ACCUMULATED_PARTIALS state and if already in the second state we change the direction of going through the range |
| next_range | 1 | When high, means partial_data going to the input sampler is valid and we go back to INITIAL_PARTIALS state, if not already in it, and the data in this range is ready for the output |
| operation_done | 1 | When high, means partial_data goint to the input sampler is valid and we go back to INITIAL_PARTIALS state, if not already in it, and the data in this range is ready for the output |

## Output Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| sample_data | 1 | High whenever the data going to the input sampler is valid |
| alu_read_en | 1 | High whenever in the ACCUMULATED_PARTIALS, low otherwise |
| Initial_partial_end | 1 | High whenever in the INITIAL_PARTIALS and phase_change, range_end or operation_end is high, low otherwise |
| accumulated_partial_end | 1 | High whenever in the ACCUMULATED_PARTIALS and range_end or operation_end is high, low otherwise |
| direction_change | 1 | High whenever phase_change is high |
| alu_input_sel | 1 | High when in ACCUMULATED_PARTIALS, low otherwise |

## FSM

When reset is asserted, we are in INITIAL_PARTIALS.

When in INITIAL_PARTIALS, the only way to go to ACCUMULATED_PARTIALS is that phase_change is high.

When in ACCUMULATED_PARTIALS, the way to go to INITIAL_PARTIALS is when range_end or operation_end is high.