# Input Sampler

## Parameters

- DATA_WIDTH.

- PARTIAL_SIZE.

- DATA_IN_SIZE = DATA_WIDTH * PARTIAL_SIZE

- NUMBER_OF_PARTIALS_PER_OUTPUT = 2.

- DATA_OUT_SIZE = DATA_WIDTH * NUMBER_OF_PARTIALS_PER_OUTPUT

## Input Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| rst_n | 1 | When asserted, delete all the registers and the outputs are = 0 |
| sample_data | 1 | When high, means the data_in is valid and should be used otherwise the data_in is discarded |
| data_in | DATA_IN_SIZE | data_in when sample_data is high, means the data_in is valid and should be used otherwise the data_in is discarded |

## Output Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| data_out | DATA_OUT_SIZE | Contains all the data sent before concatenated with the recent data_in while the oldest is the most significant bit and the data_in is the least significant bit |
| valid_out | 1 | When the counter becomes = NUMBER_OF_PARTIALS_PER_OUTPUT |

## Notes

You should make a counter that counts the number that the sample_data is asserted and save each data_in when the sample_data is asserted to a reg ( [DATA_SIZE – 1 : 0] temp [NUMBER_OF_PARTIALS_PER_OUTPUT – 2 : 0]) until the counter reaches the NUMBER_OF_PARTIALS_PER_OUTPUT and in the same clock cycle you should concatenate all the data in the reg with the data_in and put them in the output without missing a beat.