# Memory Interpreter

## Parameters

- ADDR_WIDTH.

- MEM_SIZE = 512.

- ADDR_SIZE = 9.

- RANGES_NUMBER = 2.

## Input Signals

| Signal | Number of bis | Description |
| --- | --- | --- |
| end_range | 1 | When high, it indicates that the sent address and data is the las in this range |
| address_in | ADDR_WIDTH | It's the logical address for the output data that should come from the RAM |
| change_direction | 1 | When high, you must change the direction, if you were incrementing the address decrement and vice versa |
| en | 1 | When high the saved address is incremented or decremented when low the saved address is constant |

## Output Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| out_address_out | ADDR_SIZE | The physical address for the output data that should come from the RAM |
| alu_address_out | ADDR_SIZE | The physical address for the alu data that should come from the RAM |
| output_valid | 1 | High when the address_in contains ready data |