# Synch Write Asynch Read RAM

## Parameters

- DATA_WIDTH.

- OUTPUT_SIZE.

- MEM_SIZE = 512.

- ADDR_SIZE = 9.

- DATA_SIZE = DATA_WIDTH * OUTPUT_SIZE.

[DATA_SIZE – 1 : 0] mem [MEM_SIZE – 1 : 0]

## Input Signals

- rst_n -> 1 bit -> asynchronous reset put every output = 0.

- out_read_en -> 1 bit -> when high, the out_read_data = the data in the address out_read_address.

- out_read_address -> ADDR_SIZE bit -> contains the address for the data outputed to the out port.

- alu_read_en -> 1 bit -> when high, the alu_read_data = the data in the address alu_read_address.

- alu_read_address -> ADDR_SIZE bit -> contains the address for the data outputed to the alu port.

- write_en -> 1 bit -> when high

| Signal | Number of bits | Description |
| --- | --- | --- |
| rst_n | 1 | Asynch reset put every output = 0 |
| out_read_en | 1 | when high, the out_read_data = the data in the address out_read_address else = 0 |
| out_read_address | ADDR_SIZE | contains the address for the data outputed to the out port |
| alu_read_en | 1 | when high, the alu_read_data = the data in the address alu_read_address else = 0 |
| alu_read_address | ADDR_SIZE | contains the address for the data outputed to the alu port |
| write_en | 1 | When high it makes the data on data_in written in write_address |
| write_address | ADDR_SIZE | The address which the data_in is written in |
| data_in | DATA_SIZE | The data written in the write_address |

## Output Signals

| Signal | Number of bits | Description |
| --- | --- | --- |
| out_read_data | DATA_SIZE | Data in the out_read_address if out_read_en is high else = 0 |
| alu_read_data | DATA_SIZE | Data in the alu_read_address if alu_read_en is high else = 0 |